B6x

Bluetooth 5.2 SoC

Datasheet

Specification

©2022

Disclaimer: Descriptions of specific implementations are for illustrative purpose only actual hardware implementation may differ.

。

Version History

| Version | Date | Author(s) | Description |
| --- | --- | --- | --- |
| V1.0 | 2022.12.2 | Door | Initial |
| V1.1 | 2023.1.9 | Door | Modify font format |
| V1.2 | 2023.3.27 | Door | Update chip’s package type and pin num |
| V1.3 | 2023.6.27 | Door | Update chip’s package |
| V1.4 | 2023.6.30 | Door | Update chip’s package and electrical characteristics |
| V1.5 | 2023.7.3 | Door | Update electrical characteristics |
| V1.6 | 2023.7.3 | Door | Update chip’s package |
| V1.7 | 2023.7.14 | Door | Update chip’s package |
| V1.8 | 2023.7.28 | Door | Update QFN20 Pins |
| V1.9 | 2023.8.24 | Door | Update QFN32 and QFN20 Package |
| V2.0 | 2023.10.20 | Door | Update GPIO’s description |
| V2.1 | 2023.11.17 | Door | Update SOP16’s package |
| V2.2 | 2024.1.10 | Door | Delete SOP8 |
| V2.3 | 2024.1.16 | Door | Add eSOP8 Package |
| V2.4 | 2024.5.7 | Door | Update eSOP8’s Flash Size |
| V2.5 | 2024.5.10 | Door | Update I/O port characteristics |
| V2.6 | 2024.5.14 | Door | 1. Fixed the error that the TX/RX current in the electrical was reversed 2.Update Chip power off current |
| V2.7 | 2024.5.30 | Door | Update Electrical Specifications |
| V2.8 | 2024.6.11 | Door | Update QFN32 and QFN20 Package(add pin spacing) |
| V2.9 | 2024.10.10 | Door | Update QFN20 Package |
| V3.0 | 2024.10.29 | Door | Update Reference Design Cap_C1 Value |
| V3.1 | 2024.12.24 | Door | Update SPI Master and SPI Slave speed |
| V3.2 | 2025.1.10 | Door | Update chip’s operate temperature range |
| V3.3 | 2025.5.14 | Door | Add QFN32 with 512KB Flash |
| V3.4 | 2025.7.29 | Door | Update Electrical Specifications |
| V3.5 | 2025.12.5 | Door | Update QFN32 and QFN20 Package |
| V3.6 | 2025.12.30 | Door | Update eSOP8 Package |
| V3.7 | 2026.4.21 | Door | 1.Update maximum UART baud rate 2.Update capacitance value of C5 in reference design |

Table of Contents

1	General Description	5

1.1	Overview	5

1.2	Features	5

1.3	Block Diagram	9

2	Pin Assignments	10

2.1	B6x QFN Package	10

2.1.1	QFN20-B63	10

2.1.2	QFN32-B66/B68	11

2.2	B6x SOP Package	12

2.2.1	SOP16-B62	12

2.3	eSOP8-B61	12

3	Peripheral Blocks	13

3.1	GPIO	13

3.2	Cross Switch Connect	13

3.3	UART	13

3.4	I2C	14

3.5	SPI Master	14

3.6	SPI Slave	15

3.7	Timer	15

3.8	ADC	16

3.9	Smart DMA	17

4	I/O Multiplexing and Cross Switch Connect	18

4.1	Multiplexed Signals	18

4.2	Cross Switch Connect Map	19

5	Electrical Specifications	20

6	Schematic	22

7	Ordering Information	23

8	Package Information	24

8.1	QFN20 3x3x0.75x0.4-P0.25 mm package	24

8.2	QFN32 4x4x0.75x0.4-P0.3 mm package	24

8.3	SOP16 package	25

8.4	eSOP8 package	25

- General Description

  - Overview

B6x is a very low power, high performance and highly integrated Bluetooth 5.2 BLE + 2.4G Proprietary solution, designed for operation over the 2360 to 2520MHz ISM frequency band.

B6x is manufactured using advanced CMOS low leakage process, which offers highest integration, lowest power consumption, lowest leakage current and reduced BOM cost while simplifying the overall system design.

  - Features

- BT 5.2 + 2.4GHz-Proprietary SoC

  - Supported data rates: 1Mbps、2Mbps

  - -96 dBm sensitivity @1Mbps BLE mode

  - -92 dBm sensitivity @2Mbps BLE mode

  - TX power: -40 dBm to 6 dBm

  - Single-ended antenna output

  - Integrated balun

  - Support BLE Mesh

- RISC 32-bit MCU

  - High Performance (64MHz)

- SMART 8 Channels DMA Controller

- CACHE

  - CACHE Support Range：4KB

- USB

  - USB1.1 full speed：12Mbps

  - Supports up to 5 endpoints, and endpoints 1~4 support simultaneous sending and receiving

  - All endpoints’ FIFO depth is 64 bytes

- On-Chip Memory

  - 4KB ROM

  - Max 32KB SRAM

  - eSOP8(SIP 128KB QSPI FLASH)

  - SOP16(SIP 128KB QSPI FLASH)

  - QFN20(SIP 256KB QSPI FLASH)

  - QFN32(SIP 256KB/512KB QSPI FLASH)

- Clock Generation Unit

  - Crystal Oscillator Supporting 16MHz(XTAL)

  - Internal Resonant Crystal (IRC) 16MHz +/- 2%

  - Internal programmable Ultra Low Power RC(RC32K)

  - Internal HF DPLL48M/64M clock

  - Support external injection clock

- Real Time Clock (RTC)

  - Operates from Ultra Low Power Technology

  - Operates from Internal Ultra Low Power RC

- Watch Dog Timer

  - Operates from Internal Low Power WDT Crystal

- Flex Timer Module

  - Advanced 16-bit Counter/Timer (1X) (ATMR)

  - Common 16-bit Counter/Timer (1X) (CTMR)

4 channels Compare Match with PWM / Capture / Compare

  - Basic 16-bit Counter/Timer (1X) (BTMR)

- Serial Interfaces

  - UART (2X) support RS485/IrDA/ISO7816-3, Speed up to 3.76Mbps AHB

  - SPI Master(1X), speed up to 24Mbps

  - SPI Slave(1X), speed up to 32Mbps

  - I2C-bus (1X) compatible 2-wire master/slave,

Support Fast-mode with data rates up to 400Kbps

- Analog-Digital Converter

  - 10-bit ADC, 1Mbps , 10 input channels

  - MIC function，support 8K Sample rates

- General Purpose I/Os

  - Supports up to 20 GPIOs,

each GPIO supports interrupt and wake-up functions.

- Power Management Controller

  - Integrated Power-On-Reset (POR)

  - Integrated Low-Voltage-Detect (LVD) ，Default voltage threshold 1.65V

- Voltage range

- Operating voltage range: 1.8 V to 5.2 V (device-specific)

- Power Consumption

  - On-chip DC/DC and LDO regulators with automated low current modes

  - 44.7μA/MHz (@3.3V with 16M RC)

  - Deep sleep Mode，70 μA @3.3V

  - Power off Mode，1μA @3.3V

  - 7.2mA Tx peak current in BLE/2.4G mode (@1.8V 0dBm)

  - 10.3mA Rx peak current in BLE/2.4G mode (@1.8V)

- ESD 4500V

- Operating temperature range: -40 °C to 85 °C (-40 °C to 105 °C available upon order specification)

  - Block Diagram

- Pin Assignments

This section describes the pin assignment for the different package types.

  - B6x QFN Package

    - QFN20-B63

    - QFN32-B66/B68

  - B6x SOP Package

    - SOP16-B62

  - eSOP8-B61

- Peripheral Blocks

  - GPIO

The general-purpose input/output (GPIO) is configured as a control interface with up to 20 I/O pins (package-dependent). Up to 20 pins can be accessed and controlled through one port, and each GPIO can be accessed individually.

- 14 GPIO s(QFN20)

- 20 GPIOs (QFN32)

- 10 GPIOs (SOP16)

- 3 GPIOs(eSOP8)

- Configurable output drive strength

- Internal pull-up and pull-down resistors

- Wake-up from high or low level triggers on all pins

- Trigger interrupt on state changes on any pin

  - Cross Switch Connect

SPI/UART/I2C can be selected on all GPIOs

  - UART

- Full duplex asynchronous communication

- Support single-line half duplex communication

- 16-byte TX and RX FIFOs

- Support speed up to 3.76M bps

- Support auto-baudrate

- Support DMA

- Support CTS and RTS

- Support IrDA SIR, RS-485, ISO/IEC7816-3, LIN, Modbus

  - I2C

- Master and Slave modes

- Fast mode speed up to 400k bps

- 7-bit and 10-bit addressing mode

- 8-bit TX/RX Buffers

- Hardware PEC (Packet Error Checking) generation and verification with ack

  - SPI Master

The Serial Peripheral Interface (SPI) is a synchronous serial data communication protocol that operates in full duplex mode.

- Supports SPI Serial Peripheral Interconnect Master Protocol

- Support SPI different mode：0、1、2、3(with CPOL and CPHA)

- Support full duplex, receive only and transmit only mode

- Master Mode speed upto 24Mbps

- Provides separate 8-layer depth transmit and receive FIFO buffers

- Supports DMA transfer

- Support transmit over interrupt

- Support Configure MSB/LSB first

  - SPI Slave

The Serial Peripheral Interface (SPI) is a synchronous serial data communication protocol that operates in full duplex mode.

- Supports SPI Serial Peripheral Interconnect Master Protocol

- Support SPI different mode：0、1、2、3(with CPOL and CPHA)

- Support full duplex mode

- Slave Mode speed upto 32Mbps

- Provides separate 8-layer depth transmit and receive FIFO buffers

- Supports DMA transfer

- Support transmit over interrupt

- Support Configure MSB/LSB first

  - Timer

There are 3 Counters/Timers:

- Advanced Timer (x1): Support 4 Independent channels for Input capture, Output compare, PWM and Single pulse output; dead-time; break function; ETR function

- Common Timer (x1): Support 4 Independent channels for Input capture, Output compare, PWM and Single pulse output; ETR function

- Basic timer (x1): Support 4 Independent channels for Input capture, Output compare, PWM and Single pulse output

  - ADC

10-bit precision ADC with 10 input channels, sampling rate up to 1 Mbps.

- Support work clock can be divided by 1~ 1024

- Support hardware calibration mode

- Support single conversion mode or continuous mode

- Support the DMA read data function

- Support ATMR1 trigger SADC sampling mode

- Support Channel rotation function

- MIC function support 8K Sample rates

- Support measure power voltage

- Support up to 8 channels single end input

- Data width is 10-bit, and effective bit width is up to 9-bit

  - Smart DMA

- 8 Channels DMA

- each DMA channel has a programmable priority level

- each priority level arbitrates using a fixed priority that is determined by the DMA channel number

- supports multiple transfer types:

  - memory-to-memory

  - memory-to-peripheral

  - peripheral-to-memory

- the number of transfers in a single DMA cycle can be programmed from 1 to 1024

- I/O Multiplexing and Cross Switch Connect

  - Multiplexed Signals

| PIN |  |  |  | NAME | Default | Function 0 | Function 1 | Function 2 | Function 3 | Function Analog |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| eSOP 8 | SOP 16 | QFN 20 | QFN 32 |  |  | GPIO | CSC | SPECIAL | TIMER | Analog |
|  | 16 |  |  | GND | -- | -- | -- | -- | -- | -- |
|  |  |  | 4 | VDD42 | -- | -- | -- | -- | -- | -- |
| 7 | 7 | 3 | 5,9 | VDD33① | -- | -- | -- | -- | -- | -- |
| 8 | 1 | 5 | 7 | RFP | -- | -- | -- | -- | -- | -- |
|  | 4 | 6 | 10 | RSTn/PA19 | RSTn | PA19 | CSC | -- | -- | -- |
|  |  | 7 | 11 | PA18 | GPIO | PA18 | CSC | -- | CTMR_CH4 | -- |
|  | 5 | 8 | 12 | PA17 | GPIO | PA17 | CSC | -- | CTMR_CH3 | -- |
|  |  | 9 | 13 | PA16 | GPIO | PA16 | CSC | -- | CTMR_CH2 | ANAIO6 |
|  |  |  | 14 | PA15 | GPIO | PA15 | CSC | -- | CTMR_CH1 | ANAIO5 |
|  |  |  | 15 | PA14 | GPIO | PA14 | CSC | -- | ATMR_ETR | ANAIO4 |
|  | 6 | 10 | 16 | PA13 | GPIO | PA13 | CSC | -- | ATMR_CH3N | ANAIO2 |
| 4 |  |  | 17 | PA12 | GPIO | PA12 | CSC | -- | ATMR_CH2N | ANAIO1 |
|  |  |  | 18 | PA11 | GPIO | PA11 | CSC | -- | ATMR_CH1N | ANAIO0 |
|  | 3 | 11 | 19 | PA10 | GPIO | PA10 | CSC | -- | ATMR_CH4P | ANAIO9 |
|  | 2 | 12 | 20 | PA9 | GPIO | PA9 | CSC | -- | ATMR_CH3P | ANAIO8 |
|  | 8 | 13 | 21 | PA8 | GPIO | PA8 | CSC | -- | ATMR_CH2P | ANAIO7 |
| 2 | 9 | 14 | 22 | PA7/DM② | GPIO | PA7 | CSC | -- | ATMR_CH1P | ANAIO6 |
| 3 | 10 | 15 | 23 | PA6/DP③ | GPIO | PA6 | CSC | -- | ATMR_BK | ANAIO5 |
|  |  |  | 24 | PA5 | GPIO | PA5 | CSC | -- | CTMR_CH4 | ANAIO4 |
|  |  |  | 25 | PA4 | GPIO | PA4 | CSC | -- | CTMR_CH3 | ANAIO3 |
|  |  | 16 | 26 | PA3/MICIN | GPIO | PA3 | CSC | -- | CTMR_CH2 | MICIN |
|  |  | 17 | 27 | PA2/MICBIAS④ | GPIO | PA2 | CSC | -- | CTMR_CH1 | ANAIO2 |
|  | 11 | 18 | 28 | PA1/SWDIO | SWDIO | PA1 | CSC | SWDIO | CTMR_ETR | ANAIO1 |
|  | 12 | 19 | 29 | PA0/SWCLK | SWCLK | PA0 | CSC | SWCLK | -- | ANAIO0 |
| 1 | 13 | 2 | 30 | VDD12 | -- | -- | -- | -- | -- | -- |
| 5 | 14 | 20 | 32 | XO16M_O | -- | -- | -- | -- | -- | -- |
| 6 | 15 | 1 | 1 | XO16M_I | -- | -- | -- | -- | -- | -- |

- The QFN32 package has two VDD33 pins: PIN5 is VDD33_1 and PIN9 is VDD33_2. VDD33_1 and VDD33_2 are connected internally.

- PA6 default as gpio, if you want it use as USB DP, you need to close IE, OE, PU, PD etc.…

- PA7 default as gpio, if you want it use as USB DM, you need to close IE, OE, PU, PD etc.…

- PA2 default as gpio, if you want it use as MICBIAS, you need to close IE, OE, PU, PD etc.…

| NAME | EXPLANATION |
| --- | --- |
| PA00~PA19 | GPIOs (High Level) |
| SWCLK | Debug Clock Pin |
| SWDIO | Debug Data Pin (High Level) |
| DP | USB Data Positive |
| DM | USB Data Minus |
| MICBIAS | MIC output voltage |
| MICIN | MIC signal input |
| RFP | RF Antenna Posedge |
| XO16M_O | Crystal oscillator 16M Output |
| XO16M_I | Crystal oscillator 16M Input |
| CTMR_CH1~ CTMR_CH4 | 4 Channels Common Timer1 PWC Input / PWM Output |
| ATMR_ETR /CTMR_ETR | Advanced Timer/Common Timer External Trigger |
| ATMR_BK | Advanced Timer1 Break Input |
| ATMR_CH1P~ ATMR_CH4P | 4 Channels Advanced Timer Positive PWC Input / PWM Output |
| CSC | GPIO Function Multiplexing |
| AINIO0~AINIO9 | ADC Input |
| VDD12 | Digital core voltage output 1.2V |
| VDD33 | voltage input 3.3V |
| VDD42 | voltage input 4.2V |

  - Cross Switch Connect Map

The Cross Switch Connect (CSC) controls the function of each GPIO pin in a highly flexible way by allowed to connect many functions like the UART, SPI, and I2C functions to any pin on GPIO Function 3. These functions are called movable functions and are listed in the following Table.

- Electrical Specifications

| Name | Parameter（Condition） | Min | Typical | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| Power Supplies |  |  |  |  |  |
| VDD33 | 3.3v voltage input | 1.8 | 3.3 | 3.6 | V |
| VDD42 | 4.2v voltage input | 2.7 |  | 4.2 | V |
| TEMP | Temperature |  | -40 | 25 | 85 |
| I/O port characteristics |  |  |  |  |  |
| VIH | High Level (VDDIN = VDD33/VDD42/VDD50) |  | 0.7 |  | VDD33 |
| VIL | Low Level (VDDIN = VDD33/VDD42/VDD50) |  | 0.3 |  | VDD33 |
| VOH | High Level (VDDIN = VDD33/VDD42/VDD50) |  | 0.7 |  | VDD33 |
| VOL | Low Level (VDDIN = VDD33/VDD42/VDD50) |  | 0.3 |  | VDD33 |
| Iout | GPIO output drive strength (VCCIO = 3.3V) |  | 3 | 6 | mA |
| Rpu | GPIO Weak pull-up equivalent resistor |  | 64 |  | KΩ |
| Rpd | GPIO Weak pull-down equivalent resistor |  | 68 |  | KΩ |
| Rusb | USB DP pull-up equivalent resistor |  | 1.5 |  | KΩ |
| Current Consumption |  |  |  |  |  |
| IVDD | Sleep Mode1 - The core and peripherals are powered down; all register and SRAM data are lost. - Only all GPIOs are supported for wake-up. - After wake-up, the program starts execution from address 0. |  | 1 |  | uA |
| IVDD | Sleep Mode2 - The core and peripherals are powered down; all register and SRAM data are lost. - Supports wake-up via all GPIOs and RTC. - After wake-up, the program starts execution from address 0. |  | 1.7 |  | uA |
| IVDD | Sleep Mode3 - The core and peripherals are powered down; all register and SRAM data are lost. - 8 KB of non-volatile SRAM retains data. - Supports wake-up via all GPIOs and RTC. - After wake-up, the program starts execution from address 0. - Retains BLE, peripheral, and register data for fast restoration of the pre-sleep state. |  | 2 |  | uA |
| IVDD | Sleep Mode4 - The core and peripherals remain powered on; all register and SRAM data are preserved. - Wake-up from all GPIOs and RTC is supported. - After wake-up, program execution resumes from the point where sleep occurred. |  | 70 |  | uA |
| IVDD | RX mode, BLE & 2.4G mode, 100% ON (@1.8V) |  | 7.2 |  | mA |
| IVDD | TX mode, BLE & 2.4G mode, 100% ON (@1.8V) |  | 10.3 |  | mA |
| IVDD | Advertising mode: interval 500ms (@1.8V 0dBm) |  | 51 |  | uA |
| IVDD | Link mode: interval 500ms (@1.8V) |  | 38 |  | uA |
| Normal RF Condition |  |  |  |  |  |
| FOP | Operating Frequency | 2360 |  | 2520 | MHz |
| FXTAL | Crystal Frequency |  | 16 |  | MHz |

- Schematic

| Designator | Value | Description | Footprint |
| --- | --- | --- | --- |
| C1 | 10nF | Capacitor, X7R, ±10% | 0402 |
| C2, C3 | NC |  |  |
| C4 | 2.2pF | Capacitor, X7R, ±10% | 0402 |
| C5 | 10uF | Capacitor, X7R, ±10% | 0402 |
| C6 | 100nF | Capacitor, X7R, ±10% | 0402 |
| X1 | 16MHz | XTAL SMD 3225, 16MHz, Cl=9 pF, 40Ω, ±10ppm | Seam Seal 3225 |

- Ordering Information

| Ordering Code | FLASH (bytes) | Package | Size(mm) | Carrier Type | Pack Quantity |
| --- | --- | --- | --- | --- | --- |
| B61 | 128KB | eSOP8 | 4.9x3.9 | TUBE | 10000 |
| B62 | 128KB | SOP16 | 9.9x3.9 | TUBE | 5000 |
| B63 | 256KB | QFN20 | 3x3 | Tape and Reel | 5000 |
| B66 | 256KB | QFN32 | 4x4 | Tape and Reel | 5000 |
| B68 | 512KB | QFN32 | 4x4 | Tape and Reel | 5000 |

- Package Information

  - QFN20 3x3x0.75x0.4-P0.25 mm package

  - QFN32 4x4x0.75x0.4-P0.3 mm package

  - SOP16 package

  - eSOP8 package